GVT3 VS ICELAKE VECTOR API PERF

Since Vector API benchmarking kernels are single threaded hence what matters most is CORE and RAM.

Thus, we are selecting instances with similar RAM and storage configurations.

GVT3: (C7G.2xlarge: 8 VCPU, 16 GB RAM, 32GB SSD]

ICELAKE: (M7i.xlarge: 4 VCPU, 16 GB RAM, 32 GB SSD)

ROTATE PERFORMANCE:

Vector Size 256 bits:

ICX:

GVT3:

Observations:

- Rotate is an integral operation, we do not perform rotation over floating point vectors.

- Apart from Byte level rotation, X86 scores above GVT3 for all other types.

- Unlike X86, ARM does not have a direct rotate instruction for integer and long types.

- ARM always degenerates rotates into Shift left, right and Or operations. X86 does so for sub-word types.

- ARM has single instructions of shift left/right for all the types. ARM vector instruction encoding explicitly encodes lane size, thus same operations can be performed over vectors of varying length and lane sizes.

- X86 does not have a direct shift instruction for byte vector. Thus, we promote each byte lane to word and then perform shift left/right operations. Unpacking-packing b/w byte and words adds to the path length and hence reduces overall throughput.